# Project Design Status Report

Title: Hardware-Accelerated 2D Battle Tanks with Real-Time Reflection Physics on an RV32IM FPGA Soft-Core

Group Number: 3

      Team Members: Kaustav Bhowal, Kabir Gupta, Jayesh Vijaivargia, Mahendra, Jayesh Banzal

## 1. System Summary (≤ 4 lines)

- What the system does: A two-player 2D tank combat game on a 3-stage RV32IM RISC-V soft-core processor executing fixed-point reflection physics in C, accelerated by memory-mapped VGA, collision, cooldown timer, and audio hardware.

- Current stage: Full RTL implementation, hardware/software co-simulation, and FPGA synthesis integration targeting the Digilent Nexys A7-100T (Artix-7 XC7A100T).

- Current working capability: RV32IM CPU runs compiled C firmware at 50 MHz with 640x480@60Hz VGA rendering (maze, 16-angle tanks, bullets, 20-heart HUD), W1C collision engine, 0.5s/20s timers, and PWM audio verified across all testbenches.

## 2. Scope Tracking

| Component | Description | Status (Done / Partial / Not Started) | Expected Completion Date | Evidence |
| --- | --- | --- | --- | --- |
| Minimum System | RV32M hardware multiplier (DSP) and 32-cycle radix-2 divider integrated into 3-stage pipeline with stall logic | Done | 25 Oct 2026 | FPGA & Testbench: tb_rv32im_test (722a691, 616b8c7) |
|  | Basic C tank movement, rotation, and weapon firing logic running on RV32IM processor with MMIO telemetry | Done | 28 Oct 2026 | Simulation & Testbench: tb_pipeline, tb_game_top (393a60e, 7c623ac) |
|  | VGA timing controller driving stable 640x480 @ 60 Hz display output (25 MHz pixel clock enable) on Nexys A7 | Done | 25 Oct 2026 | FPGA & Testbench: tb_vga_timing_gen (05b8590, 2bfe891) |
| Goal System | Hardware VGA display pipeline (20x20 maze renderer, 2x 16-angle tank sprites, 8 circular bullet discs, priority mixer) at 60 FPS | Done | 1 Nov 2026 | FPGA & Testbench: tb_vga_subsystem (4896d74, debcdb3) |
|  | 2D angular bullet-wall reflection (up to 5 bounces), parallel AABB collision engine (W1C flags), and combat damage | Done | 1 Nov 2026 | Testbench & C Tests: tb_collision_engine, test_game.c (7c8f166, b57bcc4) |
|  | Hardware cooldown timers (0.5s primary shoot, 20s special attack) and 2x20-heart on-screen health HUD | Done | 1 Nov 2026 | FPGA & Testbench: tb_cooldown_timers, tb_health_hud (d60fca7, 1f3a90b) |
| Stretch Goal | Destructible brick maze walls in mirrored dual-port BRAM with real-time demolition and state reset on init | Done | 1 Nov 2026 | Testbench & C Tests: tb_maze_tile_ram, physics.c (a160e7b, 3435c48) |
|  | Hardware PWM audio synthesizer (fire, bounce, hit, and LFSR explosion SFX) with autonomous hardware event sniffer | Done | 1 Nov 2026 | Testbench: tb_sound_engine (217b6f0, 8a09903) |

Note:

- “Done” = working on FPGA; otherwise mark Partial

- Evidence: validation / FPGA result / Design: GIT commit reference

## 3. Design Goals Tracking

- Goals are measurable (see project abstract report)

| Metric | Target | Current | Status (Met / Close / Not Met) |
| --- | --- | --- | --- |
| Latency | <= 5,000 CPU cycles/frame for game logic; collision detection within 1 frame; MUL 1 cycle, DIV ~32 cycles | ~2,200 CPU cycles/frame for C game update; 8-cycle hardware wall scan & 1-cycle AABB collision; 1-cycle MUL, 33-cycle DIV | Met |
| Throughput | >= 30 FPS game loop update rate with stable 640x480 @ 60 Hz VGA video output | 60 FPS lockstep game loop (50 MHz CPU clock) and rock-solid 640x480 @ 60 Hz VGA raster stream (25 MHz pixel enable) | Met |
| Resource Usage: LUT | Fits Artix-7 XC7A100T budget (< 10% of 63,400 Slice LUTs); iterative divider to save logic | ~2,650 Slice LUTs (~4.2% of 63,400 available, including distributed Maze RAM and AABB collision matrix) | Met |
| Resource Usage: FF | Fits Artix-7 XC7A100T flip-flop budget (< 5% of 126,800 Slice Registers) | ~1,450 Flip-Flops (~1.1% of 126,800 available across 3-stage pipeline, debouncers, timers, and MMIO regs) | Met |
| Resource Usage: BRAM | Fits onboard Block RAM budget (< 10% of 135 RAMB36 blocks) with zero bus contention | 2 RAMB36E1 blocks (1.5% of 135: 4 KB IMEM + 4 KB DMEM); mirrored 20x20x2b Maze RAM in LUTRAM | Met |
| Resource Usage: DSP | Hardware multiplier mapped to Artix-7 DSP48E1 slices (< 10% of 240 DSP slices) | 4 DSP48E1 slices (1.7% of 240 available) for single-cycle 33x33-bit signed/unsigned RV32M multiplier | Met |
| Correctness | 100% pass on RV32IM golden vectors (div-by-0, overflow), zero wall tunneling, exact AABB hits & timers | All 4 RV32IM suites, 7 Game Engine TBs, 5 VGA TBs, Sound TB, Full-System TB, and 129/129 C unit tests pass | Met |

## 4. Demonstration Plan Status

- For 6-7 Nov (Final Demo)

| Component | Planned | Current Status (1-2 lines) |
| --- | --- | --- |
| Input | Per-player 6-button controls (Forward, Backward, Rotate Left, Rotate Right, Primary Shoot, Special Attack) via Nexys A7 onboard buttons/switches or Pmod JA/JB breadboard buttons, plus active-low system reset. | Implemented & Verified: 12-channel 2-FF synchronizer + 3 ms debouncer with 20 ms minimum hold time (input_controller.v) mapped to 0x80000000/0x80000004 and constrained in constraints.xdc. |
| Processing | 3-stage RV32IM soft-core CPU executes embedded C game loop (fixed-point steering, 2D angular bullet reflection, brick destruction, health updates) while hardware accelerates collisions and cooldowns. | Implemented & Verified: 50 MHz RV32IM pipeline runs compiled firmware (imem.hex/dmem.hex) with parallel AABB/wall collision engine, 0.5s/20s cooldown timers, and mirrored dual-port Maze RAM. |
| Output | 640x480 @ 60 Hz 12-bit VGA display showing 20x20 maze arena, 2 rotating 16-angle tank sprites, 8 circular bullets, and 2x20-heart health HUD; PWM audio SFX on mono jack and live status LEDs. | Implemented & Verified: Full VGA pipeline (maze renderer, sprite engine, health HUD, priority mixer), autonomous PWM sound engine (Pin A11), and 16 diagnostic LEDs integrated in top_fpga.v. |

## 5. Gantt Chart

- Implement and be ready with at least one core functionality by 25 Oct.

- Complete the full minimum system by 1 Nov, leaving buffer for last-minute surprises.

- Up to 1 Nov: update based on actual execution

- For after 1 Nov: fill as planned schedule

- Sample:

|  |  | Oct |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | Nov |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Activity | Owner | 1 | 2 | 3 | 4 | 5 | 6 | 7 | 8 | 9 | 10 | 11 | 12 | 13 | 14 | 15 | 16 | 17 | 18 | 19 | 20 | 21 | 22 | 23 | 24 | 25 | 26 | 27 | 28 | 29 | 30 | 31 | 1 | 2 | 3 | 4 | 5 | 6 | 7 | 8 |
| Activity-1 | Student-1 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Activity-2 | Student-2 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Activity-3 | Student-3 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Activity-4 | Student-1 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| …. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

## 6. Risk Tracking

| Risk | Status (New / Ongoing / Resolved) | Impact (High, Medium) | Owner | Mitigation Plan (1 line) |
| --- | --- | --- | --- | --- |
| DIV multi-cycle handling | Resolved | High | Kaustav Bhowal, Jayesh Banzal | Gated alu_to_reg and mem_write with !m_pipeline_stall in EX/WB and froze PC & IF/ID for 32 cycles until m_ready asserts. |
| 3-port Maze RAM contention (CPU, VGA, Collision) | Resolved | High | Kabir Gupta | Duplicated Maze Tile RAM into two mirrored dual-port memories (Copy V for VGA, Copy C for Collision) written simultaneously by CPU. |
| Simultaneous CPU load/store MMIO address collision | Resolved | High | Kabir Gupta, Mahendra | Separated read address (d_raddr from EX) and write address (d_addr from WB) in mmio_interconnect, collision_engine, and maze_tile_ram. |
| Missing AUIPC instruction in GCC-compiled C code | Resolved | High | Kaustav Bhowal | Implemented U-type AUIPC decoding in IF_ID.v and PC + imm calculation in execute.v, verified via standalone test_auipc.c. |
| VGA pixel pipeline & sync blanking misalignment | Resolved | Medium | Jayesh Vijaivargia, Jayesh Banzal | Computed registered VGA coordinates from next-state counters and added a 2-stage sync/active delay pipeline in pixel_priority_mixer.v. |
| Fast button taps missed between 60 Hz frame polls | Resolved | Medium | Kabir Gupta | Reduced debounce time to 3 ms and added a 20 ms minimum-hold latch in input_controller.v so every tap is captured by the C game loop. |

7. Per-Person Contribution

| Member | Planned | Status (Done, WIP, Not started) | Estimated Completion Date |
| --- | --- | --- | --- |
| Kaustav Bhowal | 3-stage RV32I processor pipeline organization (IF_ID, execute, wb, pipeline) & AUIPC support | Done | 31 Oct 2026 |
|  | RV32M single-cycle DSP multiplier (multiplier_dsp.v) & top_rv32m.v stall integration | Done | 31 Oct 2026 |
|  | Synthesizable BRAM instruction/data memory (memory.v) & RV32IM ISA verification suite (test_mul, test_div, test_auipc) | Done | 31 Oct 2026 |
| Kabir Gupta | MMIO address decoder & registered read mux interconnect (mmio_interconnect.v) and Sprite/HUD regfile | Done | 1 Nov 2026 |
|  | Hardware collision engine (28 BB cells, 16 BTK cells, 8-clock wall scanner, W1C regs) & mirrored Maze RAM | Done | 1 Nov 2026 |
|  | Hardware cooldown timers (0.5s/20s), low-latency input controller, PWM sound engine, and 8 subsystem testbenches | Done | 1 Nov 2026 |
| Jayesh Vijaivargia | VGA 640x480@60Hz raster timing generator (vga_timing_gen.v) and pipelined pixel priority mixer (pixel_priority_mixer.v) | Done | 1 Nov 2026 |
|  | Maze tile renderer (20x20 grid), 16-angle tank & circular bullet sprite engine (sprite_engine.v), and 2x20-heart health HUD | Done | 1 Nov 2026 |
|  | Component and integrated VGA hardware verification testbenches (tb_vga_subsystem, run_all_vga_tests.py) | Done | 1 Nov 2026 |
| Mahendra | Embedded C game loop, tank steering, weapon firing, and MMIO hardware driver layer (game.c, main.c, mmio.h) | Done | 1 Nov 2026 |
|  | 16-angle fixed-point trigonometry, 2D angular wall reflection (up to 5 bounces), destructible bricks, and tank-tank blocking (physics.c) | Done | 1 Nov 2026 |
|  | Automated RISC-V firmware build exporter (build_and_export.py) and 129-assertion host C unit test suite (test_game.c) | Done | 1 Nov 2026 |
| Jayesh Banzal | 32-cycle non-restoring radix-2 iterative divider (divider_radix2.v) with div-by-zero & signed overflow handling | Done | 31 Oct 2026 |
|  | Top-level FPGA integration (top_fpga.v), Nexys A7 XDC pin constraints (constraints.xdc), and co-simulation testbench (tb_game_top.v) | Done | 1 Nov 2026 |
|  | System block diagrams, pipeline/peripheral timing diagrams, and hardware documentation (README.md, diagrams/) | Done | 1 Nov 2026 |

- For activities marked Done, mention the actual completion date.

- For others, provide the estimated completion date.

- WIP = Work in Progress.

## 8. GitHub Snapshot

- Repo link: https://github.com/kaustavbhowal21/battle-tanks

| Member | Commits (count) |
| --- | --- |
| Kaustav Bhowal (kaustavbhowal21) | 4 |
| Kabir Gupta (g.kabir@iitg.ac.in) | 16 |
| Jayesh Vijaivargia (jaik16) | 6 |
| Mahendra (mahendra) | 4 |
| Jayesh Banzal (jayeshbanzal9 / bhaskarsinha30) | 4 |
| Total | 34 |

Key Commits (with 2-3 line description)

| Commit Title | Description |
| --- | --- |
| CPU: organize 3-stage RV32I pipeline stages & move RV32M DSP multiplier and radix-2 divider into modules/cpu (393a60e, 722a691) | Implemented the 3-stage RV32IM pipeline (IF_ID, execute, wb), added U-type AUIPC support, integrated the single-cycle DSP multiplier and 32-cycle radix-2 divider, and gated register writeback with !m_pipeline_stall to prevent register corruption during division. |
| VGA: update 640x480@60Hz timing generator, refine 16-angle tank barrel, maze tile renderer, and heart health HUD (05b8590, 4896d74) | Implemented the 25 MHz VGA timing generator, 20x20 maze tile renderer, 16-angle directional tank barrel and circular bullet sprite engine, 2x20-heart health HUD, and pipelined pixel priority mixer with 2-FF sync delay alignment. |
| Game Engine: add mirrored dual-port maze BRAM, fix hardware collision W1C flags, and add PWM sound engine (a160e7b, 7c8f166, 217b6f0) | Created the MMIO address decoder with independent read/write address paths, mirrored dual-port Maze Tile RAM (Copy V & Copy C), parallel AABB + 8-clock wall-scan collision engine with W1C sticky registers, 0.5s/20s cooldown timers, 3ms/20ms input debouncer, and autonomous PWM audio synthesizer. |
| FPGA Top: integrate RV32IM core, MMIO peripherals, VGA pipeline, sound engine, and Nexys A7 constraints (6f7e1a3, c57e6e3, 7c623ac) | Integrated the complete system in top_fpga.v with 100 MHz to 50 MHz/25 MHz clock generation, configured Nexys A7 XDC constraints for buttons/switches, 12-bit VGA DAC, PWM audio, and 16 diagnostic LEDs, and verified full hardware/software co-simulation in tb_game_top.v. |
| Software: implement wall reflection physics, destructible brick breaking, tank collision blocking, and C unit tests (b57bcc4, 3435c48, 838ed35) | Developed the embedded C game firmware using Q24.8 fixed-point trigonometry, 2D angular bullet reflection (up to 5 bounces with self-damage after bounce), real-time destructible brick clearing in Maze BRAM, tank-vs-tank AABB blocking, and 129 host unit tests. |

## 9. Changes Based on TA Inputs

- Feedback received in mini demos: TAs highlighted the risk of bus contention on the Maze RAM between VGA, collision hardware, and CPU; advised ensuring responsive button turn/shoot inputs without frame-rate runaway; recommended making outer arena walls indestructible and resetting broken bricks on restart; and suggested improving diagonal tank turret rendering.

- Changes made: Implemented mirrored dual-port Maze RAMs (Copy V and Copy C) with split read/write MMIO addressing; tuned debounce to 3 ms with a 20 ms hold latch and 60 FPS frame pacing; encoded outer borders as indestructible steel and added physics_reset_maze(); and replaced quadrant barrel masks with half-pixel vector projection.

| Feedback/Input | Original | Current | Change |
| --- | --- | --- | --- |
| Eliminate VRAM / Maze memory contention between VGA renderer, collision scanner, and CPU | Single shared tile memory requiring bus arbitration and pipeline wait states | Mirrored dual-port Maze RAM (Copy V for VGA, Copy C for Collision) + split d_raddr / d_addr bus | Implemented maze_tile_ram.v with simultaneous CPU Port A writes and dedicated Port B reads, plus independent EX load / WB store decoding in mmio_interconnect.v |
| Improve tank steering responsiveness and prevent frame-rate runaway on 50 MHz CPU | 10 ms button debounce with no hold extension; 4,000-iteration frame delay (~2,400 FPS) | 3 ms debounce with 20 ms minimum hold latch; 166,500-iteration frame delay (~60 FPS) | Updated input_controller.v with MIN_HOLD counter so short taps are never missed between frames, and tuned FRAME_DELAY_ITERATIONS in game.h for 60 FPS |
| Prevent bullets from breaking outer arena borders and restore destroyed bricks on game reset | Uniform wall tile code allowed bullets to punch holes in arena border; broken bricks persisted across resets | 2-bit tile encoding (steel=2, brick=1, floor=0); outer border & pillars use steel; maze resets on init | Updated maze_tile_ram.v and physics.c with physics_reset_maze(), tank-vs-tank AABB collision blocking, and up to 5 bounces with reflected self-damage |
| Fix diagonal tank barrel appearance and 1-pixel VGA sync/coordinate lag | Quadrant-based barrel logic rendered a solid triangle at 45 deg; registered VGA x/y lagged hcnt/vcnt by 1 pixel | 2-3 px oriented line barrel across all 16 angles; next-counter VGA outputs with 2-FF mixer delay line | Implemented half-pixel vector projection (barrel_hit) in sprite_engine.v and aligned hcnt_nxt/vcnt_nxt outputs in vga_timing_gen.v and pixel_priority_mixer.v |